============================

** BRINGING UP THE SYSTEM **

============================

INTRODUCTION

------------

The following intructions should be followed when attaching

an XCOMP SMS/SMR controller to a CDC Phoenix hard disk drive.

DRIVE INSTALLATION

------------------

1. Follow instrunctions in the CDC Phoenix Hardware Maintenance Manual

for unpacking the drive.

2. Configure the drive for 64 sectors per track. This is accomplished

by setting the DIP switches on the Servo-Course PWA (Slot #3) as

follows: Switches 1-6 ON (depressed to the left), Switch 7 OFF

(depressed to the right), and Switch 8 ON (depressed to the left).

See section 3.10.1 of the maintenance manual for further details.

3. Set the LOC/REM switch to the LOC position (to the left) on the I/O

PWA in Slot #1. This allows the operator to control power sequencing

of the drive.

CONTROLLER INSTALLATION

-----------------------

1. Set the controller I/O address so that it does not conflict

with any other I/O in your computer. See the XCOMP SMS Users

Manual for proper switch setting.

2. Change the equate statement CBASE in all programs to reflect

the I/O address used in Step #1. Assemble all the programs

using the Digital Research macro assembler MAC.

3. Install the controller in your system. Do so with power off!

4. Connect the short 26 pin ribbon cable between the two boards.

5. Connect both the 60 & the 26 pin cables from the controller

to the drive. Pin 1 of both cables on the controller are

connected to Pin 1 on the drive connectors. Pin 1 is denoted

by a triangle on the connectors and by the red wire on the

ribbon cable.

TESTING

-------

Run the following programs in sequence:

1. XSBT. This will test the buffer memory of the controller. It also

ensures proper I/O address decoding and the ability of the host

processor to transfer data to/from the controller.

2. CDCFMT. Formats the drive. This needs to be done before data can be

written onto the drive. Tests the ability of both the controller and

the drive to read & write data. Note that this program overwrites any

previous data.

3. CDCRWT. Writes known data patterns into every sector of the disk. Reads

back the data and verifies it. Note that the program overwrites any

previous data.

5. At this point you are ready to bring up CP/M on your hard disk.

####################################################################