--- DOCUMENTATION ON  DRIVEC2.OBJ ---

THIS UNIT PROVIDES LOW LEVEL ACCESS TO THE CORVUS DRIVE

THRU "HOOKS" IN THE CORVUS I/O DRIVER.  IT SHOULD WORK ON

ANY CORVUS CONSTELLATION ][ SYSTEM UNDER MSDOS 1.1 OR 2.X.

THIS SPECIFIC UNIT HAS BEEN ASSEMBLED TO SUPPORT PROGRAMS

WRITTEN WITH THE MICROSOFT/IBM PASCAL COMPILER.

THIS UNIT IMPLEMENTS  9  PROCEDURES:

INITIO

BIOPTR		- CONST. ][

SETSRVR		- CONST. ][

FINDSRVR	- CONST. ][

NETCMD		- CONST. ][

CDRECV = DRVRECV

CDSEND = DRVSEND

THE CALLING PROCEDURE IN PASCAL IS :

CDSEND (VAR st : longstring )

THE FIRST TWO BYTES OF THE STRING ARE THE LENGTH

OF THE STRING TO BE SENT OR THE LENGTH OF THE

STRING RECEIVED.

function INITIO	: INTEGER

THE FUNCTION RETURNS A VALUE TO INDICATE THE STATUS OF

THE INITIALIZATION OPERATION.  A VALUE OF ZERO INDICATES

THAT THE INITIALIZATION WAS SUCCESSFUL.  A NON-ZERO VALUE

INDICATES THE I/O WAS NOT SETUP AND THE CALLING PROGRAM

SHOULD NOT ATTEMPT TO USE THE CORVUS DRIVERS.  THIS FUNCTION

MUST BE EXECUTED FIRST BEFORE ANY OTHER ROUTINES IN THIS UNIT

CAN BE USED.

function BIOPTR	: INTEGER

THE FUNCTION RETURNS A 16 BIT POINTER TO THE "CORTAB"

BIOS TABLE IN THE CORVUS "BIOS" DRIVERS.  THIS ROUTINE

SHOULD NOT BE EXECUTED BEFORE A SUCCESSFUL USE OF THE

"INITIO" ROUTINE (ABOVE).  NOTE:  THE RETURNED VALUE IS

RELATIVE TO "SEGMENT" ZERO, AND A RETURNED VALUE OF ZERO

INDICATES THAT THE "CORTAB" TABLE COULD NOT BE FOUND.

function SETSRVR ( srvr : integer): INTEGER

THE FUNCTION RETURNS THE "BOOT SERVER" NETWORK ADDRESS.

IF THE INPUT PARAMETER IS LESS THAN  255 ( BUT NOT NEGATIVE ),

IT WILL BE TAKEN AS A RESET OF THE DEFAULT SERVER # WHEN

USING THE  SEND & RECIEVE  ROUTINES.  IF IT IS GREATER THAN 255

OR NEGATIVE, NO CHANGE OF THE DEFAULT SERVER # WILL BE MADE.

NOTE: THE DEFAULT SERVER # IS AUTOMATICALLY SET TO THE

BOOT SERVER # WHEN THE  "INITIO" FUNCTION IS EXECUTED.

function FINDSRVR : INTEGER

THE FUNCTION RETURNS THE NETWORK ADDRESS OF A VALID DISK SERVER.

IF THE RETURNED VALUE IS GREATER THAN 63 OR NEGATIVE, THE COMMAND

FAILED TO FIND A SERVER ( THE FLAT CABLE CARDS WOULD DO THIS ).

function CARDID : INTEGER

THE FUNCTION RETURNS THE CORVUS INTERFACE CARD TYPE ( 0 - OMNINET,

1 - FLAT CABLE ).

function NETCMD ( VAR inp, VAR out: longstring) : INTEGER

THE FUNCTION IS USED TO SEND/RECIEVE DATA TO A NETWORK SERVER.

STRING  inp  SPECIFIES THE COMMAND TO SEND TO THE SERVER,

AND STRING  out  IS WHERE ANY RETURNED DATA WILL BE PLACED

( THE STRING LENGTH OF  out  WILL NOT BE CHANGED BY THIS

OPERATION UNLESS THE COMMAND FAILED- IN WHICH CASE THE LENGTH

WILL BE SET TO ZERO).  THE VALUE OF THE FUNCTION WILL BE

RETURNED AS  ZERO  IF THE OPERATION WAS SUCCESSFUL, AND

NON-ZERO IF IT FAILED.

NOTE: THE SERVER # USED WILL BE THE "BOOT SERVER" # UNLESS

THE DEFAULT IS CHANGED BY THE  "SETSRVR" CMD.

###########################################################################################################