This  letter contains the information about the DSI-32  board.

The  information  below  is  purely  the  pin  out   information.

Generally  it  is correct but the area around the right hand  top

has changed a bit. The information follows:

Gnd   33   34   35   36   37        These no.s refer to ICU

o    o    o    o    o    o         This is the header at the

o    o    o    o    o    o         top of the board.

/RST /WR  /RD  /ADS CTTL  /TSO

The  general purpose interface socket is actually a socket  where

the  optional NS32202 ICU (interrupt controller unit)  goes.  The

ICU pin out is shown below:

Pin   Desc     Pin   Desc     Pin   Desc     Pin   Desc

-------------------------------------------------------

1   Int 15     11   Int 0      21  /CS       31  /WR

2   /INT       12   D7         22  A2 *      32  CTTL

3   ST1        13   D6         23  A3 *      33  Int 1

4   Int 14     14   D5         24  A4 *      34  Int 3

5   Int 12     15   D4         25  A5 *      35  Int 5

6   Int 10     16   D3         26  A6 *      36  Int 7

7   Int 8      17   D2         27  /RST      37  Int 9

8   Int 6      18   D1         28  GND       38  Int 11

9   Int 4      19   D0         29  NC        39  Int 13

10  Int 2      20   GND        30  /RD       40  VCC

* Ignore what the NS32032 data sheet says. We use A2 - A6 instead

of  A0  - A4 because we have a 32 bit data bus while the  ICU  is

configured to be used as a 8 bit data device.

Int  0 - Int 14 are unused an unconnected except for the  signals

that go to the header at the top of the board.

Note  that  the  /RD  /WR /CS and D0 - D7 are  probably  all  the

signals that you will require. The read and write cycle time is a

total  of  400ns of which around 170ns are the  actual  read  and

write  strobe  widths,  refer to the NS32201 TCU (timing  control

unit) for more complete data. The Port address for the /CS signal

is  FFFE00H  - FFE7EH.  Note that the address lines A2  - A6  are

latched for the entire duration of the read or write cycle.

#################################################################################