---------------------------------------------------------------------------

"THE FILE CONNECTION"

8" DISKETTE SYSTEM FOR THE IBM PERSONAL COMPUTER

COPYRIGHT FLAGSTAFF ENGINEERING 6/23/83

---------------------------------------------------------------------------

1.1 - INITIAL SYSTEM INSTALLATION

The 8" diskette system consists of an 8" diskette drive in an enclosure

and a 3' ribbon cable that connects to the IBM diskette controller card.

To install the system you must insert our modified IBM controller card in

place of the standard IBM card.  To remove the cover from the IBM PC there

are two screws in the back lower corners.  After removing these screws the

top cover slides forward.  The diskette controller card is the one with a

ribbon cable running to the internal 5" drives (the card is near the power

supply).  The card is fixed by a single screw in the top bracket.  When

removing the card be sure to disconnect the signal cable running to the 5"

drives.  This cable must be reconnected when the modified IBM card is

installed.  Be sure the card is seated properly in the I/O socket.

Unless you want to use the 8" drive as a 40 track 5" drive, it is not

necessary to change any switch settings inside the PC.  If you do want to

use the 8" drive as a 5" drive,  switch S1 inside the PC must be changed

by setting 1,7, and 8 off to tell DOS more 5" drives are installed.  The

8" drives are then called C-D when being used as 5" drives and E-F when

being used as large capacity drives under DOS 2.0.  If you do not change

the switch S1 the 8" drives are then called either B-C or C-D depending

on the number of 5" drives installed in your system.

After replacing the cover, the 3' connecting cable should be plugged

into the card and also into the 8" drive.  The cable should be installed

on the drive so that the colored edge of the ribbon cable is on bottom.

The drive should then be plugged into a 110 VAC outlet.  This completes

the physical installation of the system.

1.2 - INITIAL SYSTEM INSTALLATION CHECKOUT

To verify the 8" diskette system is working properly, several diagnostic

programs are provided to checkout the hardware.  Before you can run these

diagnostic utility programs you must either load the "DISKIO" program if

using DOS 1.1 or define a device driver called "DEVICE=BIGDISK.COM" in the

system configuration file CONFIG.SYS if using DOS 2.0.  These programs are

the I/O driver for the 8" diskette.  Next, load the diagnostic program called

"CHECK8".  The program will then prompt the operator to insert a double-sided

double-density diskette Type 2D and select either drive 1 or 2.  The diskette

should be inserted with the label on the left hand side when you are facing

the front of the drive.  Also, it is important that the diskette not be write

protected.  On 8" diskettes a write protect notch is located on the edge of

the diskette first inserted into the drive and must be covered by an opaque

tape to permit writing on the diskette.  IBM diskettes do not contain this

notch but some non-IBM diskettes will have this notch.  The program

will then perform a number of checks to verify reading, writing, seeking,

and single-double density operation.  Any errors that are found will display

a message describing the problem.  If no errors are found a message is

displayed saying the drive is OK.  This program can be run at any time to

check out the 8" diskette system but do require a scratch Type 2D diskette.

If a message is displayed saying the DRIVE IS NOT READY it is usually caused

by the cable not being connected, cable upside down where it is plugged into

the drive, or the diskette is inserted backwards.  If you have any problems or

questions about the installation, please call us.

An additional check for proper operation can be done using the "VERIFY8"

program.  This program will read 300 selected sectors from a good customer

diskette.  Since only reads are performed the diskette is not erased by

the tests.  If the diskette heads are slightly misaligned these tests will

indicate a problem by showing the number of permanent unrecoverable read

errors and also the number of temporary recoverable errors.  To run this

diagnostic program called "VERIFY8".  The program will then prompt the

operator to insert the diskette to be read and select either drive 1 or 2.

The program will then identify the diskette format and start reading the

diskette sectors from Cylinder 1 through 76 on Head 0 and 1 of the diskette.

At the end of the test (approximately one minute) the total number of errors

will be displayed.  This program can be run at any time to check out the

8" diskette system.  If any problems occur try repeating the test with a

different diskette, and if the errors persist please contact us.

As a further aid to diagnose problems with the drives we have developed

a program called "DIGITAL8" that uses a special diagnostic diskette.  This

calibration diskette is recorded with different track offsets and head

azimuth settings.  It is used to precisely measure the drive head alignment.

We use this program to checkout the drives before they are shipped to a

customer and can supply the program and alignment diskette for an additional

charge.  We can also provide a similar program called "DIGITAL5" along with

the special diagnostic diskette for use with your 5" internal drives.

As a final note, it is our policy to provide free updates of our software

releases.  We do require that you send us a blank double sided diskette and

a return envelope with your address and 60 cents postage.  We normally have

new program updates every month that consist of enhancements and corrections.

Also, we periodically release new programs that are either free or available

for a small additional charge.

1.3 - DOS 2.0 AND DOS 1.1 OPERATION

As mentioned above the programs can run under either DOS releases.  The

distribution diskette is DOS 2.0.  If you want to use the programs under

DOS 1.1 you need to copy the programs to a DOS 1.1 diskette that uses 8

sectors/track instead of the 9 sectors/track used by DOS 2.0.

To make a DOS 1.1 bootable diskette, you should boot using DOS 1.1 and

format a blank diskette using "FORMAT B:/S".  Next, reboot the system using

our DOS 2.0 distribution diskette, and copy the required programs from the

distribution diskette to the blank DOS 1.1 diskette.  The programs that need

to be copied are DISKIO.COM, and such programs as IBM8TO5.COM, etc.

It is highly recommended that you switch from DOS 1.1 to DOS 2.0.  There

are significant enhancements not only in the operating system, but also in

our support for the 8" drives such a parameter passing and 1.2 MByte files.

1.4 - COPYRIGHT INSTRUCTIONS

Our programs and documentation are protected by the 1982 Copyright Laws.

We authorize the use of our programs and documentation for a single IBM

Personal Computer.  These copyrighted materials are not to be given or sold

to any other party without the express written consent of Flagstaff

Engineering.  Our diskettes are not copy protected,  and it is permissable

to make backup copies of the diskette or save them on a hard disk.  However,

please do not give these programs to unauthorized persons.

Under the new copyright laws, a company is responsible for the actions of

their employees in stealing copyrighted material.  Therefore, make sure they

are kept in a secure manner.  If you have any questions, please call us to

make arrangements for additional use of our materials.

2.1 - 8" DISKETTE DRIVE DESCRIPTION

We have selected the Mitsubishi Thin-Line Model M2896 drives for use in our

diskette system.  These drives are 2.3" W x 8.6" H x 13.2" L and they weigh

5.5 pounds.  They require +24VDC @ .75A and +5VDC @ .75A.  We use seperate

+5 and +24 volt power supplys that can operate on either 115 VAC or 230 VAC

60HZ or 50HZ.  This permits their use at most international locations.

The Mitsubishi M2896 can read/write diskettes that conform to IBM specifications

for IBM Diskette 1, IBM Diskette 2, and the IBM Diskette 2D.  These formats

are used by all IBM machines and most other computer systems.

Specification            Single Density  Double Density

Capacity With IBM Format         0.6 Mbytes      1.2 Mbytes

Recording Density                  3408 BPI        6816 BPI

Track Density                        48 TPI          48 TPI

Number of Cylinders                      77              77

Recording Method                         FM             MFM

Data Transfer Rate            250K bits/sec   500K bits/sec

Rotational Speed                    360 RPM

Access Time Average                 91 msec

Access Time Track-Track              3 msec

Access Time Settling                15 msec

Head Load Time                      35 msec

Motor Start Time                     .5 sec

MTBF                   10000 Power On Hours

MTTR                             30 Minutes

Design Life                         5 Years

2.2 - 8" DISKETTE CABLE WIRING

The IBM diskette controller card can support a total of 4 disk drives.

The first two drives are 5" internal drives and the last two drives are

the external drives.  With the modification to the IBM controller card

the external drives can be 8" instead of 5" drives.  This cable is a

ribbon cable with a 50 pin card edge connector on the end that plugs

into the 8" drive and a 37 pin D style connector on the end that plugs

into the IBM card.  The following pins are used:

NAME           IBM PIN       DISKETTE PIN

HEAD SELECT            18                 14

DRIVE SELECT 1          9                 26

DRIVE SELECT 2          8                 28

DIRECTION              11                 34

STEP                   12                 36

WRITE DATA             13                 38

WRITE GATE             14                 40

INDEX                   6                 20

TRACK 00               15                 42

WRITE PROTECT          16                 44

READ DATA              17                 46

IN USE                N/C                 16

HEAD LOAD             N/C                 18

LOW CURRENT           N/C                  2

DRIVE SELECT 3        N/C                 30

DRIVE SELECT 4        N/C                 32

DISK 2 SENSE          N/C                 10

DISK CHANGE           N/C                 12

READY                 N/C                 22

GROUND     22,24,26,28,30     15,17,19,21,23

The 37 pin connector is a TRW-CINCH DC-37P.  The 50 pin connector is a

3M 3415-0001 and the flat cable is 3M 3365-50.

3.1 - GENERAL PROGRAM NOTES

The "FILE CONNECTION" 8" diskette system for the IBM Personal Computer

consists of a I/O device driver called BIGDISK.COM and several utility

programs that are used to exchange files between the PC and other systems.

Before running any of the utility programs, either the device driver called

BIGDISK.COM (DOS version 2.0) or the DISKIO program (DOS version 1.1) must be

loaded to provide the ability to read and write single or double density

diskettes with any sector size.  A utility programs can then be loaded

to perform one of three functions:  First, to format a diskette with the

particular parameters for exchanging files (called xxxFMT).  Second, to

copy files from an 8" diskette to a DOS volume (called xxx8TO5).  Third,

to copy files from a DOS volume to an 8" diskette (called xxx5TO8).

Presently the major systems supported with these utilitys include:

IBM machines (IBM), CP/M (CPM), Radio Shack (TRS), IBM Series/1 (EDX),

Honeywell (HON), Digital Equipment (DEC), and Allen-Bradley Numerical

Control (ALB).

In addition to the basic file exchange utility programs there are many

special purpose program that provide diagnostics (CHECK8, VERIFY8) and

dual drive system copying between two 8" diskettes (COPY8, EDXTOS34, etc.)

When most of the utility programs are loaded they will first display a

a program name message that contains the copyright date.  We have been

releasing new program versions approximately every month, and this date

is used to keep track of program updates.

The utility program can operate with either of the two supported external

8" drives which we call 1 or 2 in our programs.  If your system does not

use a hard disk, drive 1 is DOS drive 'C' and drive 2 is DOS drive 'D'.  If

your system uses special device handlers for other devices such as a hard

disk the DOS volume letter may be some other value from A-Z.

When a utility program prompts the operator for an entry such as (Y/N) or

(1/2), the program will read only a single character entry and you don't

need to press the ENTER key.  If you press the ENTER key without typing a

character, the program will default to the first character in the displayed

message options '(default/option1/option2/option3)'.  For example,  when the

program says 'DO YOU WANT EBCDIC TO ASCII CONVERSION (N/Y)' and you press the

ENTER key the program will default to NO conversion.

3.2 - DISKIO PROGRAM DESCRIPTION

The program module called DISKIO.COM is the I/O control for the diskette

drives using DOS version 1.1.  The program is loaded and permanently resides

until the system is restarted.  This program must be loaded before any of

the utility programs are run.  When it is loaded it changes the INT 13H

interrupt pointers to handle all diskette I/O calls.  No program load message

is displayed when it is running.  After the DISKIO program has been loaded,

the system should be rebooted before using the drives with DOS.

The DISKIO program is very similar to the diskette I/O control code located

in the PC's Read Only Memory with several special additions.  First, the

user can change parameters for any of the diskette drives.  All 4 diskette

drives are initially set to 512 Bytes, 26 Sectors, MFM encoding.  To change

the parameters a function request is provided by issuing an INT 13H command.

Register DL is 0-3 to specify which drive.  AH is 8 to specify a set parameter

function request.  AL is a 0-7 to set one of the eight available formats.

Once the parameters are set for a specific drive they remain until another

set parameter request is issued.  The available drive parameters are:

AL  Bytes/Sector  Sector/Track  Encoding

0           128            26       FM

1           256            15       FM

2           512             8       FM

3          1024             4       FM

4           256            26      MFM

5           512            15      MFM

6          1024             8      MFM

7          2048             4      MFM

The second special change is the addition of two new read-write commands

for the diskette drives.  A function AH=6 is used to read a deleted sector

on the diskette.  This command operates the same as the AH=2 read command

except a single sector is read with skip flag off.  This is used to read

deleted HDR1 labels on IBM 3741 diskettes.  A function AH=7 is used to

write a deleted sector on the diskette.  This command is the same as the

AH=3 write command except sectors are written with a special control

address mark.  This is used to write deleted HDR1 labels on IBM diskettes.

3.3 - BIGDISK DEVICE DRIVER PROGRAM DESCRIPTION

The program module called BIGDISK.COM is the I/O control for the diskette

drives using DOS version 2.0.  This program is very similar to the program

called DISKIO for DOS version 1.1 except it is written to be a device handler

and becomes a part of the DOS 2.0 operating system.  To include this device

handler, a special file called CONFIG.SYS must be created using the EDLIN text

editor or COPY CON A:CONFIG.SYS.  To include this device handler enter the

statement DEVICE=BIGDISK.COM.  It is also recommended that you also enter the

statement BUFFERS=4 to increase the number of disk buffers.  Additional

documentation on configuring your system is included in Chapter 9 of the DOS

reference manual.

After the device handler is loaded, a message is displayed saying it is

running.  The operator can then use the 8" drives as either DOS drives with

a 1.2 MByte capacity or with any of our utility programs.  When used as

1.2 Mbyte drives each diskette can hold 96 files and be used with most IBM

programs, assemblers, compilers, and utilities.

When using the 8" drives as DOS drives, they are usually referenced as

B,C,D,or E depending on the number of drives defined with switch S1 on the

main processor board.  For example, if the system has no hard disk with two

5" drives and one 8" drive, and the user has set S1 to 1,8=OFF and 7=ON

(three drives defined).  Drives A-B would be regular 5" drives with 360 KByte

each, Drive C would be the first 8" drive with 360 KByte, and Drive D would

also be the 8" drive with the larger 1.2 MByte capacity.  In this system,

you can format the 8" drive as a 5" drive by referencing Drive C with the

FORMAT and DISKCOPY utility programs.  You could not however, use the FORMAT

or DISKCOPY utilities with Drive D since the 1.2 MByte device handler is used.

To format an 8" diskette for use as a large capacity volume a utility

program called BIGFMT is provided.  The user must use double sided diskettes

rated for double density (IBM Type 2D).

4.1 - CHECK8 PROGRAM DESCRIPTION

The CHECK8 program is designed to diagnose problems with the diskette

system hardware.  We use this program to inspect the drives we receive from

the factory and it does a good job in identifying even marginal problems.

After laoding the "CHECK8" program, a message will be displayed.  The user

is then prompted to insert the blank diskette and to select either the first

(1) or second (2) 8" drive unit.  You should use a high quality double sided

and double density diskette 2D.  If you use a single sided diskette, the

program will still run but only Head 0 is checked.  ** Remember to use a

scratch diskette since the data is wiped out **.  The program will then

format and read verify recording with various sector sizes using both single

and double density.  After these tests the program will then format the

entire diskette to 512 byte sectors using double density.  This format is

probably the most difficult to record properly.  If the program detects an

error in formatting a track the status code will be non-zero.  The program

will retry the format of this track and normally the retry is succesful.  If

the error still occurs after 5 trys the program will display a message to

use a different blank diskette.  After the formatting is complete, the program

will then read selected sectors on the diskette.  Starting with Cylinder 1,

the program will step forward 11 Cylinders and read a single sector.  The

program will then step backward 5 Cylinders and again read a single sector.

This pattern will be repeated until Cylinder 76 is reached.  The program will

then repeat the above step using 9 forward and 4 backward, 7 forward and 3

backward, 5 forward and 2 backward, 3 forward and 1 backward, and finally 1

forward and 0 backward.

Depending on the quality of the blank diskette you use, it is possible that

a few temporary errors will occur.  If any permanent errors occur you should

retry the test to see if the errors occur at the same diskette location.  If

the error occurs at the same location it is probable the diskette has a bad

spot and you should try a different diskette.

4.2 - CHECK5 PROGRAM DESCRIPTION

The CHECK5 program is designed to diagnose problems with the internal

5" diskette hardware.  We use this program to check our drives to make sure

they are in good condition.  This program is similar to the CHECK8 program

except it works with the internal 5" drives.

4.3 - VERIFY8 PROGRAM DESCRIPTION

The VERIFY8 program is designed to diagnose problems with the diskette

system hardware.  We use this program to inspect the drives we receive from

the factory and it does a good job in identifying head alignment problems.

After loading the "VERIFY8" program, a message will be displayed.  The user

is then prompted to insert the data diskette and to select either the first

(1) or second (2) 8" drive unit.  You should use a high quality double sided

and double density diskette 2D.  If you use a single sided diskette, the

program will still run but only Head 0 is checked.  The program will first

examine Cylinder 5 to identify the diskette formatting.  The program will

then read selected sectors on the diskette.  Starting with Cylinder 1,

the program will step forward 11 Cylinders and read a single sector.  The

program will then step backward 5 Cylinders and again read a single sector.

This pattern will be repeated until Cylinder 76 is reached.  The program will

then repeat the above step using 9 forward and 4 backward, 7 forward and 3

backward, 5 forward and 2 backward, 3 forward and 1 backward, and finally 1

forward and 0 backward.

Depending on the quality of the data diskette you use, it is possible that

a few temporary errors will occur.  If any permanent errors occur you should

retry the test to see if the errors occur at the same diskette location.  If

the error occurs at the same location it is probable the diskette has a bad

spot and you should try a different diskette.

4.4 - VERIFY5 PROGRAM DESCRIPTION

The VERIFY5 program is designed to diagnose problems with the internal

5" diskette hardware.  We use this program to periodically inspect our drives

to make sure the heads are in alignment.  This program is similar to the

VERIFY8 program except only 512 byte double density operation is checked.

5.1 - BIGFMT PROGRAM DESCRIPTION

The BIGFMT program is designed to format an IBM Diskette 2D for use with

DOS version 2.0 as a 1,174,528 byte capacity diskette.  The diskette is

formatted with 15-512 byte sectors/track double density on both sides.  It

is important to use only high quality diskettes such as IBM, 3M, etc. to

avoid possible data errors.  It should be noted that double density operation

on an 8" diskette is much more critical than double density on a 5" diskette.

Before running this program, you must first have loaded either the I/O

control program "DISKIO" (DOS 1.1) or the device handler "BIGDISK" (DOS 2.0).

Next, load the "BIGFMT" program and a message will be displayed.  The user

is then prompted to insert the blank diskette and to select either the first

(1) or second (2) 8" drive unit.

The program will then start to format the diskette.  After each track is

written the program will perform a read verify to make sure the diskette

is good.  If an error occurs during the read verify the format is retried

5 times.  If the error still occurs the program will display the bad track

and end the formatting operation.  The program does not assign alternate

tracks since it is our opinion that any diskette that contains alternate

tracks is probably marginal and should not be used.

After the diskette has been formatted a message is displayed saying the

diskette is OK and showing the number of temporary errors.  The user is

then prompted to enter the volume name for the diskette.  The name can be

a maximum of 11 characters.  The operator is then prompted to format more

diskettes.  If the operator replies 'N' the program will end.

6.1 - IBMFMT PROGRAM DESCRIPTION

The IBMFMT program is designed to format an IBM Diskette 1, 2, or 2D for

use with an IBM system.  The Diskette 1 is a single sided and single density

diskette, the Diskette 2 is a double sided and single density diskette, and

the Diskette 2D is a double sided and double density diskette.  All three

diskette types contain 77 cylinders.  The file directory is located on

cylinder 00 and the data on cylinders 01-74.  Any bad diskette tracks are

assigned to alternate tracks on cylinders 75-76.  Data tracks are formatted

with different sizes and numbers of sectors.  The standard formats are:

DENSITY  SECTOR SIZE  SECTORS/TRACK  TOTAL CAPACITY  IBM MACHINE TYPES

Single     128 Bytes             26   242,944 Bytes  Almost all IBM machines

Single     256 Bytes             15   284,160 Bytes  3601,S/1,S/38

Single     512 Bytes              8   303,104 Bytes  S/1,S/32,S/34,S/38

Double     256 Bytes             26   985,088 Bytes  S/34,S/38,8100

Double     512 Bytes             15 1,136,640 Bytes  S/38

Double    1024 Bytes              8 1,212,416 Bytes  S/34,S/38

For all IBM diskettes Cylinder 0 Head 0 is always formatted as 128 Byte

sectors in single density.  The first two sectors are IPL sectors, the next

two are reserved sectors, an error map sector, another reserved sector, the

volume label, and then 19 HDR1 label sectors.  The first HDR1 label is

defined with valid diskette extents.  The remaining 18 HDR1 labels are

written as deleted sectors to indicate they are not used.  If the diskette

is double sided Cylinder 0 Head 1 contains an additional 52 deleted HDR1

labels.  Each HDR1 label points to the location of a file on the diskette.

It contains information such as the file name, starting file location,

ending file location, record size, file creation date, and file type.

After loading the "IBMFMT" program, a message will be displayed.  The user

is then prompted to insert the blank diskette and to select either the first

(1) or second (2) 8" drive unit.  The program will then prompt the operator

to specify one (1) or two (2) sides for formatting.  Next, the program will

prompt for single (S) or double (D) density recording.  Next, the program

will prompt for the sector size.  The valid sizes are 128, 256, 512, and

1024 byte sectors.  The program will then prompt the operator for either

ASCII or EBCDIC formatting.  All IBM machines use EBCDIC character codes,

but occasionaly other systems use ASCII codes which requires this option.

If you have a question about which format parameters to use for a specific

IBM machine, use the program called "IBM8TO5" to identify the format for

a diskette previously used in that particular IBM machine.

The program will then start to format the diskette.  After each track is

written the program will perform a read verify to make sure the diskette

is good.  If an error occurs during the read verify the format is retried

5 times.  If the error still occurs the program will display the bad track

and end the formatting operation.  The program does not assign alternate

tracks since it is our opinion that any diskette that contains alternate

tracks is probably marginal and should not be used.

After the diskette has been formatted a message is displayed saying the

diskette is OK and showing the number of temporary errors.  The user is

then prompted to format more diskettes.  If the operator replies 'N' the

program will end.

6.2 - IBM8TO5 PROGRAM DESCRIPTION

The IBM8TO5 program is designed to copy a file from any IBM diskette

to a DOS file.  The program can copy any of the 71 data files on the

diskette.  The DOS files will be written with different record sizes

depending on the HDR1 label parameters.  The program can copy either

unblocked or blocked-spanned records recorded in Type B, H, E, or I

data exchange formats.

After the "IBM8TO5" program is loaded, a name message will be displayed.

The user is then prompted to insert the 8" IBM diskette and to select

either the first (1) or second (2) 8" drive unit.  The program will then

read Cylinder 5, Head 0, Sector 1 to determine the diskette formatting.

If the format is found the program then checks Head 1 to see if the diskette

has two sides.  The diskette format is then displayed.  If the program can

not identify the diskette format a message is displayed saying the diskette

cannot be copied.  The user is then prompted to select the destination

DOS drive ID from A-D.  The drive can be any valid DOS volume including a

hard disk drive.  The program will then read the first 19 HDR1 labels on

the diskette and then display them for the operator.  The user is the

prompted to enter a file number from 1-19 to specify which file to copy.

If the operator enters a 99, all files in the 8" directory will be copied.

The program will then use the file extents found in the HDR1 label to

locate the data file.  The operator is then prompted for EBCDIC to ASCII

conversion.  Most 8" IBM diskettes use EBCDIC but the Personal Computer

uses ASCII.  If conversion is selected, each character is translated from

EBCDIC to its ASCII equivalent, but no provision is currently made for

excluding fields that contain binary or packed decimal numbers.

The program will then prompt the operator to save the first 8" diskette

sector as a FORMAT1 label.  On certain IBM machines such as the System/34

and System/38, a special record is written at the beginning of each file

which contains the copy parameters when the file was saved on the diskette.

This information is then used when the file is restored on a System/34 or

System/38.  Since it is not used by the PC so it should be removed during

the copy.

The program will use the 8 character name in the IBM HDR1 label with an

.IBM extension for the DOS file name.  The program will translate all

control and special characters to an @ character for the DOS file name.

If the FORMAT1 label is saved it will use an extension of .IBS.  The

program will first delete this file and then recreate the file on the

DOS volume.  The program will then read a track on the IBM diskette and

write the DOS records.  For each 8" diskette sector read, the program will

write from 1-16 128 byte DOS records.  If the file was very large, the

8" diskette may be one of several so called multi-volume diskettes.  The

program will prompt the operator to insert the next multi-volume diskette

until the last in the series has been copied.  After all the records have

been read, the program will display an end of copy message with the number

of records copied.

After a file is copied, the operator will then be prompted to select

another file number to be copied.  If the operator does not make any

entry before pressing the return key, the program will then prompt the

operator to copy files from a different diskette.  If the operator

replies 'N' the program will end.

6.3 - IBM5TO8 PROGRAM DESCRIPTION

The IBM5TO8 program is designed to copy a DOS file to any IBM diskette.

The program can copy to any of the first 19 files on the 8" diskette.

After the "IBM5TO8" program is loaded, a name message will be displayed.

The user is then prompted to insert the 8" IBM diskette and to select

either the first (1) or second (2) 8" drive unit.  The program will then

attempt to read Cylinder 5, Head 0, Sector 1 to determine the diskette

formatting.  If the format is found the program then checks Head 1 to see

if the diskette has two sides.  The diskette format is then displayed.

If the program can not identify the diskette format a message is displayed

saying the diskette cannot be copied.  The program will then read the first

19 HDR1 labels on the diskette and then display them for the operator.

The program will use the location of the last file saved on the 8" diskette

to determine the starting location for the next file.  The operator is then

prompted to enter the name of the DOS file for the copy.  The program will

then attempt to open this file.  The operator is then prompted for ASCII

to EBCDIC conversion.  Most 8" IBM diskettes use EBCDIC but the Personal

Computer uses ASCII.  If conversion is selected, each character is

translated from ASCII to its EBCDIC equivalent, but no provision is

currently made for excluding fields that contain binary or packed

decimal numbers.

The program will then prompt the operator for the DOS file name of a

FORMAT1 label to be restored as the first 8" diskette sector.  On certain

IBM machines such as the System/34 and System/38, a special record is

written at the beginning of each file which contains the copy parameters

when the file was saved on the diskette.  This information is then used

when the file is restored on a System/34 or System/38.  This special file

can be created by using the IBM8TO5 program to save a previous FORMAT1

label or by using the DEBUG utility to create a special FORMAT1 file.

The program will use the 8 character DOS file name in the IBM HDR1

label.  The program will then read the correct number of 128 byte DOS

records to write a track on the IBM diskette.  After all the records

have been read, the program will display an end of copy message with

the number of records copied.

After a file is copied, the operator will then be prompted to select

another DOS file to be copied.  If the operator does not make any

entry before pressing the return key, the program will then prompt the

operator to copy files from a different diskette.  If the operator

replies 'N' the program will end.

7.1 - CPMFMT PROGRAM DESCRIPTION

The CPMFMT program is designed to format an IBM Diskette 1, 2, or 2D for

use with a CP/M system.  The Diskette 1 is a single sided and single density

diskette, the Diskette 2 is a double sided and single density diskette, and

the Diskette 2D is a double sided and double density diskette.  All three

diskette types contain 77 cylinders.  The system boot loader is located on

cylinder 00 and the data on cylinders 01-74.  Any bad diskette tracks are

assigned to alternate tracks on cylinders 75-76.  Data tracks are formatted

with different sizes and numbers of sectors.  The standard formats are:

DENSITY  SECTOR SIZE  SECTORS/TRACK  TOTAL CAPACITY  CP/M MACHINE TYPES

Single     128 Bytes             26   242,944 Bytes  Universal CP/M standard

Single     256 Bytes             15   284,160 Bytes  Fairly Common

Single     512 Bytes              8   303,104 Bytes  Rare

Double     256 Bytes             26   985,088 Bytes  Very Common

Double     512 Bytes             15 1,136,640 Bytes  Rare

Double    1024 Bytes              8 1,212,416 Bytes  Rare

For all CP/M diskettes, Cylinder 0 Head 0 is always formatted as 128 Byte

sectors in single density.  The remaining tracks on the diskette are formatted

with the selected density and sector size.  When the first CP/M data file

is written on the diskette the file directory entry is recorded starting

normally on Cylinder 2 Head 0.  However, some systems such as NEC start the

directory on Cylinder 1 and others use Cylinder 6.  Each directory entry

contains information on the file name, the file location, and its size.

After loading the "CPMFMT" program, a message will be displayed.  The user

is then prompted to insert the blank diskette and to select either the first

(1) or second (2) 8" drive unit.  The program will then prompt the operator

to specify one (1) or two (2) sides for formatting.  Next, the program will

prompt for single (S) or double (D) density recording.  Next, the program

will prompt for the sector size.  The valid sizes are 128, 256, 512, and

1024 byte sectors.  If you have a question about which format parameters

to use for a specific CP/M machine, use the program called "CPM8TO5" to

identify the format for a diskette used in that particular CP/M system.

The program will then start to format the diskette.  After each track is

written the program will perform a read verify to make sure the diskette

is good.  If an error occurs during the read verify the format is retried

5 times.  If the error still occurs the program will display the bad track

and end the formatting operation.  The program does not assign alternate

tracks since it is our opinion that any diskette that contains alternate

tracks is probably marginal and should not be used.

After the diskette has been formatted a message is displayed saying the

diskette is OK and showing the number of temporary errors.  The user is

then prompted to format more diskettes.  If the operator replies 'N' the

program will end.

7.2 - CPM8TO5 PROGRAM DESCRIPTION

The CPM8TO5 program is designed to copy a file from any CP/M diskette

to a DOS file.  The program can copy any of the first 64 files on the

diskette.  The DOS files will be written as 128 byte records so that for

each 8" diskette sector read, from 1-16 DOS records will be written.

After the "CPM8TO5" program is loaded, a name message will be displayed.

The user is then prompted to insert the 8" IBM diskette and to select

either the first (1) or second (2) 8" drive unit.  The program will then

read Cylinder 5, Head 0, Sector 1 to determine the diskette formatting.

If the format is found the program then checks Head 1 to see if the diskette

has two sides.  The diskette format is then displayed.  If the program can

not identify the diskette format a message is displayed saying the diskette

cannot be copied.  The user is then prompted to select the destination

DOS drive ID from A-D.  The drive can be any valid DOS volume including a

hard disk drive.  The program will then ask the operator to enter the CP/M

sector skew factor.  This number is used to locate the sectors on each track

since they are not written sequentially as on IBM machines.  The standard

skew factor for 128 byte single density sectors is 6 and for most double

density diskettes is 3.  The skew factor may be different for your system.

Next, the program will prompt the operator for the directory cylinder.

For most CP/M systems, the directory is located on Cylinder 2 but newer

systems often use Cylinder 1 and system diskettes sometimes use Cylinder 6.

The program will then read the file directory and display the file name,

number, and size in 128 byte records for the operator.  The user is then

prompted to enter a file number from 1-64 to specify which file to copy.

If the operator enters a 99, all files in the 8" directory will be copied.

The program will then use the file information in the directory to locate

the data file.

The program will use the 11 character name and extension found in the

CP/M directory entry for the DOS file name.  The program will translate all

control and special characters to an @ character for the DOS file name.

The program will first delete this file and then recreate the file on the

DOS volume.  The program will then read a track on the CP/M diskette and

write the DOS records.  For each 8" diskette sector read, the program will

write from 1-16 128 byte DOS records.  After all the records have been

read, the program will display an end of copy message with the number of

records copied.

After a file is copied, the operator will then be prompted to select

another file number to be copied.  If the operator does not make any

entry before pressing the return key, the program will then prompt the

operator to copy files from a different diskette.  If the operator

replies 'N' the program will end.

7.3 - CPM5TO8 PROGRAM DESCRIPTION

The CPM5TO8 program is designed to copy a DOS file to any CP/M diskette.

The program can copy to any of the first 64 files on the 8" diskette.

After the "CPM5TO8" program is loaded, a name message will be displayed.

The user is then prompted to insert the 8" IBM diskette and to select

either the first (1) or second (2) 8" drive unit.  The program will then

read Cylinder 5, Head 0, Sector 1 to determine the diskette formatting.

If the format is found the program then checks Head 1 to see if the diskette

has two sides.  The diskette format is then displayed.  If the program can

not identify the diskette format a message is displayed saying the diskette

cannot be copied.  The user is then prompted to select the destination

DOS drive ID from A-D.  The drive can be any valid DOS volume including a

hard disk drive.  The program will then ask the operator to enter the CP/M

sector skew factor.  This number is used to locate the sectors on each track

since they are not written sequentially as on IBM machines.  The standard

skew factor for 128 byte single density sectors is 6 and for most double

density diskettes is 3.  The skew factor may be different for your system.

Next, the program will prompt the operator for the directory cylinder.

For most CP/M systems, the directory is located on Cylinder 2 but newer

systems often use Cylinder 1 and system diskettes sometimes use Cylinder 6.

The program will then read the file directory and display the file name,

number, and size in 128 byte records for the operator.  The user is then

prompted to erase the previous file entries so the next file will be number

1 starting at block 2.  Be careful if you select the Y option (the default

is NO), since all previous file are erased from the CP/M directory.

The program will then prompt the operator for the name of the DOS file.

The program will use the 11 character DOS file name in the CP/M directory.

The program will then read the correct number of 128 byte DOS records to

write a track on the IBM diskette.  After all the records have been read,

the program will display an end of copy message with the number of records

copied.

After a file is copied, the operator will then be prompted to select

another DOS file to be copied.  If the operator does not make any

entry before pressing the return key, the program will then prompt the

operator to copy files from a different diskette.  If the operator

replies 'N' the program will end.

8.1 - TRSFMT PROGRAM DESCRIPTION

The TRSFMT program is designed to format a Diskette 1 or 2D for use with a

Radio-Shack TRSDOS system.  The Diskette 1 is a single sided and single density

diskette, the Diskette 2D is a double sided and double density diskette.  All

diskette types contain 77 cylinders.  The file directory is located on

cylinder 44 and the data on cylinders 01-74.  Data tracks are formatted

with different sizes and numbers of sectors.  The standard formats are:

DENSITY  SECTOR SIZE  SECTORS/TRACK  TOTAL CAPACITY

Single     128 Bytes             26   242,944 Bytes

Single     256 Bytes             15   284,160 Bytes

Double     256 Bytes             26   985,088 Bytes

For all TRS diskettes Cylinder 0 Head 0 is always formatted as 128 Byte

sectors in single density.  The first two sectors are IPL sectors.  The program

will also initialize the TRS file directory located on Cylinder=44 Sector=1.

The directory is initialized as the entire track.

To run this program you must first have loaded the "DISKIO" I/O module.

Next, load the "TRSFMT" program and a message will be displayed.  The user

is then prompted to insert the blank diskette and to select either the first

(1) or second (2) 8" drive unit.  The program will then prompt the operator

to specify one (1) or two (2) sides for formatting.  Next, the program will

prompt for single (S) or double (D) density recording.  Next, the program

will prompt for the sector size.  The valid sizes are 128 or 256 byte sectors.

The program will then start to format the diskette.  After each track is

written the program will perform a read verify to make sure the diskette

is good.  If an error occurs during the read verify the format is retried

5 times.  If the error still occurs the program will display the bad track

and end the formatting operation.  The program does not assign alternate

tracks since it is our opinion that any diskette that contains alternate

tracks is probably marginal and should not be used.

After the diskette has been formatted a message is displayed saying the

diskette is OK and showing the number of temporary errors.  The user is

then prompted to format more diskettes.  If the operator replies 'N' the

program will end.

8.2 - TRS8TO5 PROGRAM DESCRIPTION

The TRS8TO5 program is designed to copy a file from a TRSDOS 8"

diskette to a DOS file.  The program can copy any of the first 64 data

files on the diskette.  The DOS files will be written as 128 byte records.

To run this program you must first have loaded the "DISKIO" I/O module.

After the "TRS8TO5" program is loaded, a name message will be displayed.

The user is then prompted to insert the 8" TRSDOS diskette and to select

either the first (1) or second (2) 8" drive unit.  The program will then

read Cylinder 5, Head 0, Sector 1 to determine the diskette formatting.

If the format is found the program then checks Head 1 to see if the diskette

has two sides.  The diskette format is then displayed.  If the program can

not identify the diskette format a message is displayed saying the diskette

cannot be copied.  The user is then prompted to select the destination

DOS drive ID from A-D.  The drive can be any valid DOS volume including a

hard disk drive.  The program will next read the TRSDOS file directory on

Cylinder 44 and then display them for the operator.  The user is then

prompted to enter a file number from 1-64 to specify which file to copy.

If the operator enters a 99, all files in the directory will be copied.

The program will use the 8 character name in the TRSDOS directory with a

.TRS extension for the DOS file name.  The program will translate all

control and special characters to an @ character for the DOS file name.

The operator is then prompted to change the default DOS name if desired.

The program will first delete this file and then recreate the file on the

DOS volume.  The program will then start to copy the data records.  After

all the records have been read, the program will display an end of copy

message with the number of records copied.

After a file is copied, the operator will then be prompted to select

another file number to be copied.  If the operator does not make any

entry before pressing the return key, the program will then prompt the

operator to copy files from a different diskette.  If the operator

replies 'N' the program will end.

8.3 - TRS5TO8 PROGRAM DESCRIPTION

The TRS5TO8 program is designed to copy a file to a TRSDOS diskette

from a DOS file.  The program can copy a maximum of 64 data files

to the diskette.  The DOS files will be copied as 128 byte records.

To run this program you must first have loaded the "DISKIO" I/O module.

After the "TRS5TO8" program is loaded, a name message will be displayed.

The user is then prompted to insert the 8" IBM diskette and to select

either the first (1) or second (2) 8" drive unit.  The program will then

read Cylinder 5, Head 0, Sector 1 to determine the diskette formatting.

If the format is found the program then checks Head 1 to see if the diskette

has two sides.  The diskette format is then displayed.  If the program can

not identify the diskette format a message is displayed saying the diskette

cannot be copied.  The program will next read the TRSDOS file directory on

Cylinder 44 and then display them for the operator.  The user is then prompted

to erase any previous files in the TRSDOS directory.  If the operator enters

a 'Y', the program will initialize the TRSDOS directory.  The user is then

prompted to enter the DOS file name to be copied.

The program will then start to copy the data records.  After all the records

have been read, the program will display an end of copy message with the

number of records copied.  The TRSDOS directory entry is then updated using

a special file name for the TRS file name.  The special name is required to

provide the correct hash security code.  If desired, the file can be renamed

before being processed by a TRS system.

After a file is copied, the operator will then be prompted to select

another DOS file to be copied.  If the operator does not make any

entry before pressing the return key, the program will then prompt the

operator to copy files from a different diskette.  If the operator

replies 'N' the program will end.

9.1 - CPY8TO5 PROGRAM DESCRIPTION

The CPY8TO5 program is designed to copy data from a specific location on

an 8" diskette to a DOS file.  This utility is used to copy data from a

diskette that contains an erased or invalid file directory.

To run this program you must first have loaded the "DISKIO" I/O module.

After the "CPY8TO5" program is loaded, a name message will be displayed.

The user is then prompted to insert the 8" IBM diskette and to select

either the first (1) or second (2) 8" drive unit.  The program will then

prompt the operator to enter the starting extent for the copy.  The user

must enter a two digit cylinder number (0-77), the head number (0-1), and

then the two digit sector number (1-26).  Next, the program will request

the ending extent for the copy in the same format (CCHSS).  The program will

then read the first copy sector to determine the diskette formatting.  If

the format is found the program then checks Head 1 to see if the diskette

has two sides.  The diskette format is then displayed.  If the program can

not identify the diskette format a message is displayed saying the diskette

cannot be copied.  The program will use the 8" diskette sector size for the

DOS record size.

The user is then prompted to select the destination DOS drive and file name.

The drive can be any valid DOS volume including a hard disk drive.  The program will then ask the operator to enter the CP/M

will then open this file and display an error message if the name format is

incorrect or the no room is available in the DOS file directory.  The operator

is then prompted for EBCDIC to ASCII conversion.  Most 8" IBM diskettes use

EBCDIC but the Personal Computer uses ASCII.  If conversion is selected, each

character is translated from EBCDIC to its ASCII equivalent, but no provision

is currently made for excluding fields that contain binary or packed decimal

numbers.  The program will then start to copy the 8" data records to the DOS

file,  After all the records have been read, the program will display an end

of copy message with the number of records copied.

After a file is copied, the operator will then be prompted to enter new

copy extents.  If the operator does not make any entry before pressing the

return key, the program will then prompt the operator to copy files from a

different diskette.  If the operator replies 'N' the program will end.

9.2 - CPY5TO8 PROGRAM DESCRIPTION

The CPY5TO8 program is designed to copy data from a DOS file to a specific

location on an 8" diskette.  This utility is used to copy data to a diskette

that contains an erased or invalid file directory.

To run this program you must first have loaded the "DISKIO" I/O module.

After the "CPY5TO8" program is loaded, a name message will be displayed.

The user is then prompted to insert the 8" IBM diskette and to select

either the first (1) or second (2) 8" drive unit.  The program will then

prompt the operator to enter the starting extent for the copy.  The user

must enter a two digit cylinder number (0-77), the head number (0-1), and

then the two digit sector number (1-26).  The program will then read the

first copy sector to determine the diskette formatting.  If the format is

found the program then checks Head 1 to see if the diskette has two sides.

The diskette format is then displayed.  If the program cannot identify the

diskette format a message is displayed saying the diskette cannot be copied.

The program will use the 8" diskette sector size for the DOS record size.

The user is then prompted to select the source DOS drive and file name.

The drive can be any valid DOS volume including a hard disk drive.  The program will then ask the operator to enter the CP/M

will then open this file and display an error message if the name format is

incorrect or the no room is available in the DOS file directory.  The operator

is then prompted for ASCII to EBCDIC conversion.  Most 8" IBM diskettes use

EBCDIC but the Personal Computer uses ASCII.  If conversion is selected, each

character is translated from ASCII to its EBCDIC equivalent, but no provision

is currently made for excluding fields that contain binary or packed decimal

numbers.  The program will then start to copy the 8" data records from the DOS

file,  After all the records have been read, the program will display an end

of copy message with the number of records copied.

After a file is copied, the operator will then be prompted to enter a new

copy extent.  If the operator does not make any entry before pressing the

return key, the program will then prompt the operator to copy files from a

different diskette.  If the operator replies 'N' the program will end.

---------------------- END OF PROGRAM DOCUMENTATION ------------------------