SIM09 version 0.1

-----------------

(C)opyright 1992

By: Benoit Archambault

SIM09 may be distributed freely to users. It may not be

sold  or  packaged,  either  individually, or as part of any

other product without the express  written  consent  of  the

author.  Any  financial  remunerations  may  be  paid to the

author. This product may not be changed or altered and  then

distributed  without  the  express  written  consent  of the

author.

DISCLAIMER: SIM09 is offered on an as-is-basis  without  any

guarantee  as  to  the  correct  functioning  or fitness for

specific purpose. The author believe this program to work as

described but you use the program entirely at your own risk.

The author will not  be  responsible  for  any  hardware  or

software  damage,  loss of data, or incidental or consequen-

tial damage that may result from its  use,  whether  or  not

such use is in accordance with the instructions.

Please send any inquiries to:

Universite de Montreal

a/s Benoit Archambault

C.P. 6128, Succ. A

Montreal, (Quebec)

H3C 3J7

or

archamba@ERE.UMontreal.CA

November 5, 1992

- 2 -

MENUS     Once SIM09 is started, a menu will appear  at  the

top  of  the  screen  which consists of 6 items. I

will describe each of them in the following  para-

graphs.

You can choose an item by pressing  the  right  or

left  arrows on your keyboard. Once your choice is

made just press the return  key  to  activate  the

menu.  Now  you can choose a function by using the

up or down keys and press the return key  to  exe-

cute  the  desired  function. Please note that you

can exit the menu at any time by pressing the  ESC

key.

FILE

LOAD:     You can load a  file  directly  in  the  processor

address  space,  remember  that the MC6809 address

space is only 64K.

SAVE:     Any program in the processor address space can  be

written on disk, just supply the addresses.

SAVE MACHINE:  All CPU registers can be written on disk  for

future recovery.

LOAD MACHINE:  Recover all registers from file. Please  note

that you should have saved the registers with

the previous function !

Change DIR:    Just enter the directory you wish to go to.

November 5, 1992

- 3 -

RUN

STEP      This is the default option of SIM09. Each instruc-

tion is executed step by step. After the execution

all registers are displayed at the bottom  of  the

screen.

TRACE     This option lets you execute in trace  mode.  This

means   that  each  instruction  is  executed  and

displayed one and after an other. If you bump into

an  illegal opcode, SIM09 will stop and ask you to

confirm execution of the program.

DISK      This option is not  yet  implemented  although  it

should be in the near future !!

RUN       When this option is activated the CPU is  said  to

be  in  free  running  mode.  This  option  is the

fastest in terms of number  of  instructions  exe-

cuted  per  sec.   Neither  are  the  instructions

displayed nor the registers.

November 5, 1992

- 4 -

MONITOR

EXECUTE   This function actually starts SIM09 at the address

that  it  prompts you to enter. The execution pro-

cess depends on which option you selected  in  the

RUN menu. You can exit this process by hitting the

ESC key at any time.  Please refer to the RUN menu

for more details.

DISASSEMBLE    This sure is an interesting one.  SIM09  will

disassemble  a  screen  full of instructions.

Press any key to continue or ESC to exit  the

menu.

FILL      This function lets you fill any memory block  with

an  8  bit  value.  Again, just supply the parame-

ters.

MOVE      This function lets you move a  memory  block  any-

where within the address space.

DUMP      You can dump memory on the screen in both  Hexade-

cimal  and  ASCII  format.  Again,  just enter the

parameters.

COUNTER   You can set  the  cycle  counter  to  the  desired

value. Just enter the decimal value.

REGISTERS This option lets you modify the  contents  of  the

CPU  registers.   Press  the  'n' key to go to the

Next register or 'p' to go to the previous  regis-

ter.  When  you  are  ready to modify the register

press the ENTER key and type in the new value.

MEMORY    This option lets you modify memory in  the  MC6809

address space.  After typing in the start address,

you can use the LEFT, RIGHT, UP, DOWN keys to move

about  the first 256 bytes of data. You can modify

any cell by  pressing  ENTER  and  typing  in  the

desired  Hexadecimal  value.  You  can  go to next

block by pressing the PAGE_DOWN key or PAGE_UP key

to go to the previous block.

INTERRUPTS     All CPU special vectors  are  displayed  when

this  option  is selected.  Those vectors are

located at hex.: $FFF0-$FFFF.

November 5, 1992

- 5 -

BREAKPOINTS

ADD       You can set up to 8 breakpoints.  Just  enter  the

address.

DELETE    You can delete any breakpoint,  just  specify  the

breakpoint number (1-8).

CLEAR     Clear all breakpoints.

SHOW      Lets you see the breakpoints table.

November 5, 1992

- 6 -

DOS

This menu will prompt you to enter the DOS command you  wish

to execute. Just type in ENTER if you wish to go to DOS.

November 5, 1992

- 7 -

Special I/O Functions

$FF00     - Write character

$FF01     - Coordinate X

$FF02     - Coordinate Y

$FF03     - Scroll screen

$FF04     - Clear Screen

$FF08     - Keyboard status, 0= No key

$FF09     - DATA from keyboard (8 bits)

November 5, 1992