TCA1.  Source Code on the Diskette

The source code for the CP/M installation and the tiny-c

interpreter are in the file TC.ASM.  It can be assembled

with the CP/M 1.4 standard assembler, ASM.  The patch issued

by Digital Research on 1/8/79 must be installed in ASM.

The source code on the diskette differs from the listings

in Appendix A of the Owner's Manual in these ways:

1.	The installation code is in the file TC.ASM,

but is not in Appendix A.

2.	The installation code comes first in the file,

and the interpreter immediately follows.  The

interpreter origin is set to the first page boundary

after the end of the installation code.  Presently,

this is 600H, whereas the publication origin (in

Appendix A) is 2000H.

3.	Versions may differ -- check the version number.

Usually the diskette will have the same or a more

recent version than Appendix A.

Assembling and loading the source as furnished (i.e. unchanged)

will produce exactly the TC.COM as furnished.  Before

doing anything with tiny-c, including assembling the source,

make a backup copy on a fresh diskette.  We suggest whenever

you re-assemble tiny-c you give the new COM file the name

XTC.COM, until it is thoroughly tested.  Then erase TC.COM

and rename XTC.COM.  This assures you always have a working

version of tiny-c.

TCA2.  Installing Private MC's

Private MC's are very easily installed using the source code.

Just edit the private MC code into TC.ASM.  Be sure to have a

backup before doing this.  The comment lines:

;INCLUDE links to private MC's here

and

;INCLUDE code for private MC's here

are in the source code to indicate appropriate places to put

code for private MC's.  See the Owner's Manual, Section 2.10

for more on programming machine calls.  Use of the assembler

automates several of the steps, as follows.  On pages 2-31

through 2-34 is a six step process:

Step 1 is partly done.  The JMP MYTCS, the branch code for

three private MC's, and the JMP MCESET are all there.

All that is needed is a new

CPI n

JZ  MC100n

for each new MC you install.  Put this where the

first INCLUDE comment is.

Steps 2-4 are the programming rules that must be followed.

Put the MC code where the second INCLUDE comment is.

Steps 5-6 are automated by use of the assembler.

The only precaution is to not use any symbols already used in

the TC.ASM file.  The assembler will generate a P error code if

you violate this, so it is easily caught and fixed.

If tiny-c doesn't work after installing a private MC, you have

probably violated one of Steps 2 through 4.

TCA3.  Allocation of RAM simplified.

In the Owner's Manual, Sections 6.5.2 and 6.5.3 give

instructions for allocating RAM.  This is more easily done

using the assembler.  As furnished, tiny-c will use RAM from

the end of the tiny-c interpreter (about 1700 or 1800H) to

4CF0H.  This is suitable for a 24K CP/M with the Tarbell CBIOS.

To use more memory, proceed as follows:

Determine FBASE for your sysgen of CP/M.

Locate the EFREE EQU 4CF0H statement in the source.

Change this so EFREE is equated to FBASE - 10H.

Reassemble and load.

The new TC will use all the memory up to the new EFREE, and

have reasonable allocations of the four areas and the 8080

stack.

Special allocations can still be done.  Use DDT and follow the

instructions in Sections 6.5.2 and 6.5.3.

TCA4.  Changes to the Printed Guide.

In Section C1 of the printed guide accompanying this diskette,

the file CTC.ASM should be deleted.  This was the CP/M

installation source code.  It is now part of TC.ASM.

In Section C8, the routine ifcb should be sefcb, for

set extended file control block.

Section C9 and C10 are obsolete, and the techniques described

above should be used instead.

#######################################################################################################################