.uj 0

..---!------------------!----!------------------!----!------------------!----

Super-186 PPI bit definitions

PPI-1:  [sys_ppi] Drive control, DMA mux., TOD data, Switches.

-------------------------------------------------------------------------

|        Port A		|         Port B	|	  Port C        |

|    Mode 0, output     |    Mode 0, input      |  Mode 0, input/output |

-----------------------------------------------------------------------------

|    Motor-on*          |  Centronics/IEEE-488* |    OKI-data 3         | D15

|    Select side 1/0*   |    2Mhz/2.4576Mhz*    |    OKI-data 2         | D14

|    5.25"/8"*          |    Boot jumper 1      |    OKI-data 1         | D13

| Single/Double* density|    Boot jumper 0      |    OKI-data 0         | D12

|    Driv�selec�3�    �    8Mhz/4Mhz�         � Enabl� dbl. bi� err.* | D11

|    Drive select 2*    |    Memory size 2      |    DMA mux. 2         | D10

|    Drive select 1*    |    Memory size 1      |    DMA mux. 1         | D09

|    Drive select 0*    |    Memory size 0      |    DMA mux. 0         | D08

-----------------------------------------------------------------------------

PPI-2:  [sasi_ppi] SASI data, SASI control, SASI status.

-------------------------------------------------------------------------

|        Port A		|	  Port B	|	  Port C        |

|    Mode 0, input      |    Mode 0, output     |  Mode 0, output/input |

-----------------------------------------------------------------------------

|    SASI data 8 in     |    SASI data 8 out    |    Data out enable*   | D07

|    SASI data 7 in     |    SASI data 7 out    |    SASI Attention*    | D06

|    SASI data 6 in     |    SASI data 6 out    |    SASI Reset*        | D05

|    SASI data 5 in     |    SASI data 5 out    |    SASI Select*       | D04

|    SASI data 4 in     |    SASI data 4 out    |    SASI Busy*         | D03

|    SASI data 3 in     |    SASI data 3 out    |    SASI Control*/Data | D02

|    SASI data 2 in     |    SASI data 2 out    |    SASI Input*/Output | D01

|    SASI data 1 in     |    SASI data 1 out    |    SASI Request*      | D00

-----------------------------------------------------------------------------

PPI-3:  [cen_ppi] Centronics data, Centronics control, Centronics status,

TOD address, TOD control, ROM phantom.

-------------------------------------------------------------------------

|        Port A		|	  Port B	|	  Port C        |

|    Mode 0, output     |    Mode 0, output     |  Mode 1, input/input  |

-----------------------------------------------------------------------------

|   Centronics data 8   |    OKI-hold           |   Centronics strobe*  | D15

|   Centronics data 7   |    OKI-write          | Centronics Acknowledge| D14

|   Centronics data 6   |    OKI-read           |    SASI message*      | D13

|   Centronics data 5   |    ROM-off*           |    Drive 2-sided*     | D12

|   Centronics data 4   |    OKI address 3      |  Centronics interrupt | D11

|   Centronics data 3   |    OKI address 2      |    Centronics fault*  | D10

|   Centronics data 2   |    OKI address 1      | Centronics paper empty| D09

|   Centronics data 1   |    OKI address 0      |    Centronics busy    | D08

-----------------------------------------------------------------------------

##########################################################################################