*** DEVELOP.DOC ***

Development methodes of a 68K system by the use of AX68K utility set.

1981-December

Cottage Industry

Hamamatsu city, JAPAN

1.	Program test of a small system

In the amateur, the following methodes are used sometimes. ??

Write test source in your CP/M macine by the ED.COM .

Assemble it by AX68K.

List up .PRN file by control-P and X parm.

(ex. transient command  AX68K TEST.AZX)

Or transient command AX68K TEST

and command TYPE TEST.PRN .

Write the assembled macine code by a methode

(hand writing through a panel switch etc.)

2.	PROM based system

If you do'nt have a no program input port,

you will implant the program by a PROM.

For this methode, please use ROMWR.COM

which is contained in AX68K disk set in the source form (ROMWR.ASM) .

Please rewrite this source to match with your PROM writer.

And assemble it by CP/M ASM.COM .

PROMWR.COM will translate ASCII code in .OBJ file to binary ,

and write those into your PROM.

Then run it.

3.	More powerful methode

In the PROM based system, once the program corrections occured,

you will re-write PROM or will even wait PROM erasure.

This costs a big loss time in a system development.

For this reason , the following utilities are supported

in this AX68K disk set.

XFER is the utility for transfer the object file from your CP/M macine

to the target system.

GETOBJ is the utility for get object from the XFER

and write it into the RAM in the target system at the indicated address.

For this methode, you have to make some alteration.

First, you must prepare I/O port for program tranfer.

I/O ports must be made both in CP/M macine and the target system.

This may be RS-232C or IEEE-488 or parallel port or DMA etc.

Then rewrite GETOBJ.ASM to support the I/O.

Assemble it by CP/M assembler.

And write it into PROM and settle it in the target system.

The next step, please rewrite XFER.ASM to support the I/O port

in your CP/M macine.

Assemble it by ASM.COM .

And make XFER.COM .

After those preparation ,you will have a more powerful

program implantation methode.

The program will be assembled by AX68K on your CP/M macine.

Then it's .OBJ file will be transfered by XFER.

And will be gotten by GETOBJ in your target 68K macine.

If the error was found after the execution of the transfered program,

you can correct its source by editor on your CP/M macine,

and re-transfer its object.

This process is very fast and combenient.

The transfer speed is about 10 sec./ 1 K bytes .

It can be trimed by the rewriting the GETOBJ and XFER

upto hardware response time.

4.	ASCII file transfer

If you developed the BASIC interpretter on your target macine,

you can also transfer it's source by XFER2.

XFER2 can transfer any type files on your CP/M macine

to the target 68K system.

(XFER2 can transfer even .COM files.)

The file name to be transfer must be full name with extention.

( ex. TEST.BAS etc.)

These procedure will make enable to construct file system

on your CP/M macine without disk OS in your target system.

5.	LABEL REFERENCE

The label reference table is listed by LABELP.COM .

Please use LABELP just after the AX68K execution if want to.

If you want to develop simbolic debugger on your 68K system,

you may use label table of the assembled program.

The label table remained in the RAM of your CP/M macine,

just after the AX68K execution.

Therefore you can use this table by other program

if you did not destroied it by other large sized program.

Regarding to the label table position (address) in the RAM

refer to SLIST.DOC ,LABELP.ASM, and AX68K.ASM etc.

Transfer of the label table to the target system,

alter the XFER2.

If you done above procedure, you will have a more

powerful development system for the target 68K system.

6.	RELOCATABLE OBJECT

Unfortunately, the AX68K is not macro assembler,

so the program must written by the absolute addressing.

But if you use your technique on software,

MC68000 will be able to generate completely relocatable object

even under the absolute addressing assembler.

For example, use LEA operation or relative effective addressings

or address register index addressings etc.

The MC68000 power will enable you to write relocatable program

in almost various kinds of programs.

7.	AX68K's ability

Almost every programs will may be written by AX68K.

According to my experience, 10K programs (object size)

was developed with not so much inconvenience.

( I developed full BASIC (30K objectsized) interpretter, DDT like monitor debugger,

self assembler, self dis-assembler, floating point package ,

CP/M like DOS ,Editor ,PIP ,STAT etc.

Those are about 100~200k source sized each.

AX68K's ability was enough to write those programs.)

But more big complex programs are not the AX68K's scope.

The elementary principle of the AX68K development

is to take off the target 16 bit system.

Once after you waked up your system by AX68K,

other more comfortable and powerful macro assemblers and languages

and operating systems will lead you into a happy new 16 bit world.

Good luck !

CP/M is a trademark of Digital Research, Inc.

MC68000 is a trademark of MOTOROLA, Inc.

################################################