Vfeature may be used to install high capacity, high performance disk drives on

systems which will be used as a 3-COM file server.

The 3-COM file \ETHERSHR\ETHERSTD.BAT must be edited to change "CIOSYS 16" to

"CIOSYS 25".

If the file server is to be booted in STANDARD mode, the maximum volume size

is 32 megabytes.  There is no volume size limitation in DEDICATED mode.

3-COM PLUS:  File server boot volume is limited to 32 megabytes.  There is no

limitation on other volumes.  After Vfeature has been installed, edit the

CONFIG.SYS file on the hard disk to contain device=ansi.sys under our device

driver.  CONFIG.SYS should then look similar to the following example:

device=fixt_drv.sys

device=ansi.sys

buffers=15

files=20

Copy DOS ANSI.SYS to the root directory and reboot the sytem.  Then install

3-COM PLUS as described in their documentation.