MKT:@(#) 64700-17534 A.00.08 64700 SW UTIL                            06Jan95

This  file  has  a  list of files  on  the  distribution  floppies  and

directions  for installing them manually.   These directions should  be

used if the 'install' program will not work on your computer, or if you

have  special  requirements  not satisfied by  the  'install'  program.

Please  try the 'install' program first.   If you are not familiar with

the concepts of DOS tree structured directories, environment variables,

or  execution  paths,  please refer to the DOS manual  for  instruction

prior to attempting to install your software manually.

Please note that not all of these  floppies may have been  supplied with

this product, and that the name of the FIRMWARE floppies may differ (eg:

ANALYSIS FIRMWARE); however, the directory  structures and file contents

are still  applicable.  In particular, the EMULATION  FIRMWARE  floppies

are supplied with the human interface software for each emulator.

CONTENTS OF DISTRIBUTION DISCS:

64700 SW UTIL Disc 1:

|       (root directory)

-------------

/ install.exe

/  setup.txt

/   readme.doc  (this file)

/

HP64700     (sub-directory)

-----------

/ |      \

/  |       \

/   |     TABLES   (sub-directories)

/    |   ----------

/     |    64700TAB    (Default emulator device table)

/      |

/     -----

/       BIN        (sub-directory)

/         |

/      -------------

/       PROGFLASH.EXE   (Flash EEPROM update utility)

/

LIB     (sub-directory)

-------

npf.X (downloaded to HP64700 by PROGFLASH.EXE)

lpf.X (downloaded to HP64700 by PROGFLASH.EXE)

FIRMWARE Disc 1:

|      (root directory)

-------------

/

/

HP64700         (sub-directory)

------------

/

/

UPDATE

--------

647xx.X   (Firmware for HP647xx)

647xx.cfg  (Flash EEPROM programming configuration)

The  general idea of installation is to get the files needed  onto  the

hard  disc.   The files must be placed into sub-directories on the hard

disc as follows:

<tld>       (Top level directory, your choice of

------------    name. Install defaults to: \HP64700)

/      |     \

/       |      \

BIN        |     TABLES      (sub-directories)

-------------   |   ----------

PROGFLASH.EXE  / \   64700TAB

/   \

/     \

/       \

LIB     UPDATE

-------    -------

npf.X     647xx.X

lpf.X    647xx.cfg

...

647yy.X

647yy.cfg

In addition to creating the directory structure on the target hard disc,

and copying the files required, the environment variable HP64700 must be

assigned  the path  name of the top  level  directory.  The  environment

variable  HPTABLES  must  be  assigned  the  path  name  of  the  TABLES

directory, this path will always be the TABLES  subdirectory of your top

level directory  (<tld>).  These variables are used by  PROGFLASH.EXE to

find the required  files.  The DOS 'set'  command is used to  initialize

these  variables (ex:  set  HP64700=C:\HP64700).  You will probably want

to place commands to set these  varibles into your  'autoexec.bat'  file

for execution on boot up.

You should also add the 'BIN' sub-directory to your execution 'PATH' to

allow  execution  of the development programs while you are  developing

software in your own directories.  See the DOS command 'path' for usage

of this command.  (ex: PATH=\;\com;\HP64700\BIN)  You will want to edit

or add this command to your 'autoexec.bat' file also.

Follow these steps to install your software manually:

o   Decide upon a directory name for your top level directory. <tld>

o   Build the directory structure if it does not already exist.

md <tld>

md <tld>\BIN

md <tld>\LIB

md <tld>\TABLES

md <tld>\UPDATE

o   Copy  the  'PROGFLASH.EXE'  file  from  the  64700  SW UTIL  disc 1

'\HP64700\BIN'   directory  to  the  file:  <tld>\BIN\PROGFLASH.EXE

(example:  copy A:\HP64700\BIN\PROGFLASH.EXE C:\HP64700\BIN)

o   Copy the '*pf.X' files from the 64700 SW UTIL disc 1 '\HP64700\LIB'

directory to the files: <tld>\LIB\*pf.X.

(example: copy A:\HP64700\LIB\*.X C:\HP64700\LIB)

o  Copy   the   '*.X'   and   '*.cfg'   files   from   FIRMWARE   discs

'\HP64700\UPDATE' directory to the directory:  <tld>\UPDATE

(example:  copy A:\HP64700\UPDATE\*.* C:\HP64700\UPDATE)

o   If you do not already have an emulator device table file (64700TAB)

that you have  set up and want to  keep,  copy the  default  device

table file:  \HP64700\TABLES\64700TAB from the 64700 SW UTIL disc 1

to the file: <tld>\TABLES\64700TAB.

(example: copy A:\HP64700\TABLES\64700TAB C:\HP64700\TABLES)

o   Edit  your  autoexec.bat  file  to add  or edit the 'set  HP64700=',

'set HPTABLES=' and 'PATH=' commands as discussed above.

o   Reboot the computer to set up environment and check operation.