Documentation Date:    16 June 1987

Program Version:       1.82 PCDOS

Program Version date:  870226

Program Author:        Dwight W. Gordon

HDPARK.COM is a program designed to place the heads of your hard drive(s)

on the innermost cylinder available.  It will function properly on a system

that is BIOS compatible with an IBMPC as far as the hard drive BIOS is

concerned.  This means that the hard drive card should be one of the XEBEC

cards.  The program has worked properly on my IBMPC clone and on my Zenith

158.  It has NOT been tested on a system with a Western Digital hard drive

controller card.  It has NOT worked to my satisfaction on an IBM AT.

This program has been conservatively designed.  It first requests the size

of the drive (from the BIOS).  It increments the cylinder count and performs

a seek.  If the seek is successful, it continues to increment until the seek

is not successful.  It then backs up one cylinder.  It then performs a read.

If the read is successful, then it displays a successful park and the

cylinder on which it occurred.  The read is necessary because a seek does not

check that the head actually got to the proper cylinder.  (I have seen

situations where a 10Meg drive had successfully seeked to cylinder 611 if

the parameters for the drive were incorrectly programmed.)  This program

has occasionally displayed that a drive had been UNsuccessfully parked

when it had parked successfully, but I have not seen the reverse true.

I suggest that you park your hard drive(s) any time you plan to turn off

system power.  If you have only one hard drive on your system, then do not

be concerned that hard drive 1 parked unsuccessfully.  If it had parked

successfully, then there would be need for concern.

This program parks the drive(s) and waits for keyboard input.  If no

input takes place within about a minute, the program blanks the screen in

order to prevent screen burn-in.  Once the drive(s) is (are) successfully

parked you may remove system power (turn off the switch) or you may return

to the operating system by striking any alphanumeric key on the keyboard.

This program is better that most I've seen in that it:

1.  Allows you to get to the operating system if you should desire,

2.  Gives you visual feedback as to where your hard drive(s) parked.

Most 10Meg drives will park on cylinder 305.  Most 20Meg drives will park

on one of cylinders 611, 614 or 615.  If at any time your drive parks at

a "funny" place, either discontinue to use this program or run your

diagnostics.

If at any time your hard drive makes "funny" sounds when it is parking,

then do not use this program.

Program information for this archive:

Archive:  HDPARK18.ARC

Name          Length    Stowage    SF   Size now  Date       Time    CRC

============  ========  ========  ====  ========  =========  ======  ====

-870616              0   Packed    00%         0  16 Jun 87  08:47   0000

-VERSION.182         0    -----    00%         0  16 Jun 87  10:55   0000

HDPARK.DOC        ????  Crunched   ??%      ????  16 Jun 87  ??:??   ????

HDPARK.XXX        1124  Crunched    5%      1067  26 Feb 87  14:16   DED6

*total       4    ????             ??%      ????

Programming Assumptions:

1.  There must be no hardware faults in the hard disk support system.

2.  The program must be uncorrupted.

3.  Your hard drive subsystem meets the aforementioned restrictions.

Licensing Assumptions and Restrictions:

1.  This program remains the exclusive property of the author. Copyright

1987, All rights reserved.

2.  It may be distributed freely in the form provided by this .ARC file.

AT NOT TIME MAY IT BE SOLD OR INCLUDED WITH ANY OTHER PROGRAMS THAT ARE

SOLD.  This includes (but is not restricted to) freeware/shareware service

companies (eg. "$5.00 a disk" types).  If you wish to make arrangements

for this type of arrangement, then you must obtain prior written permission

from the author.

3.  It may not be modified in any shape or form without the expressed

written permission of the author.  DO NOT USE THIS PROGRAM if you suspect

that the program has been modified.  There is the potential to mess up

your hard drive if the program becomes corrupted.

4.  This program is provided "as is."  No warranties are provided.  The

author takes no responsibility for any damage done as a result of this

program.  Use the program at your own risk.  My suggestion is that PRIOR

to your using any program like this that you BACK UP YOUR HARD DRIVE.

Comments and Suggestions should be addressed to:

Dwight W. Gordon

Kansas State University

Department of Electrical

and Computer Engineering

Durland Hall

Manhattan, Kansas 66506

To provide for your safety (and to encourage your reading this document

first) the .COM file has been included in this archive with the name

"HDPARK.XXX".  To make it a usable program execute the following command

from the DOS prompt:

"REN HDPARK.XXX HDPARK.COM"