THE MICROPERIPHERAL CORPORATION

4800 BAUD MODEM

The 300-4800 baud modem  from  The MicroPeripheral  Corp.

features a  unique system of  data  transfer (patent  pending)

which employs digital/analog  converters.  Basically the input

port  serial  data  stream  is  converted  to an  analog  tone

equivalent  determined  by  the  number  of ones or  zeros  in

sequence.   At the receiving end, the tones are reconverted to

their digital equivalents  and  fed to  the serial output port

of the modem.

When no data is being transmitted,  there are no tones on

the  line.   This  permits  multiple  users  in  parallel  for

networking  applications.  The Clear-To-Send  (CTS)  line goes

high  whenever  data  is  received,  thereby  preventing  data

collisions.

TECHNICAL SPECIFICATIONS

GENERAL

Power Source-    115 vac, 60 Hz input supplies 12 vac at 50

ma. from UL listed power source

Power Consumpt.- 1 watt maximum

Dimensions-      7.5" W x 5.5" L x 2" D

Weight-          1.75 pounds (.784 kg)

Ambient-         Operating temperature 0-50 degrees C (32-

112 degrees F), operating humidity 10- 90

relative w/o condensation.

Compatibility-   New Stoner format, not Bell 208 compatible

Data Rate-       300 baud to 4800 baud

Operating Mode-  Half duplex, on two wire, full duplex on

four wire lines.

Operating Format-Asynchronous binary serial.

Modulation-      D to A/A to D conversion.

Line Impedance-  600 ohms nominal.

Line Standard-   Two wire direct distance dialing (DDD)

Bell Telephone and similar networks.

Interface-       Six foot telephone cord terminated in

modular plug (supplied).  Mates with RJ-11

modular jack.  Standard RS-232 type DB-25

(female) connector for EIA RS232C.

Ringer Equiv.    0.0B

Modulation Freq. Transmits and receives various frequencies

between 300 and 3000 Hz.

Sensitivity-     Up to -48 dbm (adjustable).

Line Level-      -10 dbm at 600 ohms (per FCC Part 68

permissive.

AUTODIAL/AUTOANSWER (Option)

Dialing-         Make-break  keying actuated  by  RTS  or  DTR

line.

Answer Wait-     15 seconds minimum from off-hook.

Carrier Detect-  15 seconds minimum from answer.

Data Delay-      2 seconds minimum for carrier acquisition.

Ring Detect-     Software selectable, default one ring.

Line Acquisit-   Actuated by DTR line.

#ition.

Ring Detect-     Software sel