The Bye programs have a note that I included on the polarity

of the jump instructions after an IN from a port.  IF problems

exist in the execution of the program, you should confirm that

the logic for your particular UART/USART/ACE is correctly

reflected. For example in the ringwt routine:

RINGW2	EQU	$+OFFSET

IN	RPORT	;GET THE STATUS

ANI	P2RDET	;RINGING?

JZ	RINGWT	;NO, WAIT

THE 'JZ' MAY BE REPLACED WITH A 'JNZ' TO REFLECT THE USART/ACE

IMPLEMENTATION.

Check your system documentation to determine the correct

logic.  Again this applies only to conditional jumps or

calls after an ' IN ' from a port...

Thom Quick

###############################################################################################################