# Running Code Composer – Real Time Tutorial for C24xx DSPs

- Open project Real24xx.mak :

## Menu bar -> Project -> open

Type:

### C:\tic2xx\c2000\tutorial\realtime\ Real24xx.mak

Click -> open

Note: You may need to browse for this file if you installed Code Composer in a directory other than the default c:\ti2xx directory.

- Edit x24x_app.h to select the appropriate DSP as shown below –

x240		.set	0	; C/F240

x241		.set	0	; F241

x242		.set	0	; C242

x243		.set	1	; C/F243

x2402		.set	0	; C/F2402

x2404		.set	0	; C2404

x2406		.set	0	; C/F2406

x2407		.set	0	; F2407

The above example selects TMS320F243. In order to choose TMS320LF2407 device set 1 for x2407 and put 0 for x243.

- Build Project (Assemble and link code)

Menu bar > Project > Rebuild All

Output window status should read:

“Build complete, 0 Errors, 0 Warnings”

- Download tutorial example code to EVM board

Menu bar > File > Load Program

Select Real24xx.out from the dialog box and click on Open.

Note: If you receive a loading error, verify you have the target jumpers set for running out of RAM with watchdog disabled (JP5 = 2-3, JP6=1-2).

- Reset the DSP

Menu bar > Debug > Reset DSP

- To Run code emulation in real-time mode

Type go MON_GO in Command Window and execute -

Click Real Time Mode from Debug

Menu bar > Debug > Real Time Mode

After selecting Real Time Mode run the software - 2

Menu bar > Debug > Run

The DSP should now be running in Real Time Mode.

#### DSP is now running in Real Time mode

# Adding Watch Windows

- Add Watch windows:

Menu bar > View > watch Window

- Declare variables or peripheral registers to watch:

Note: Cursor should be on watch window

Right-mouse click on Insert New Expression

(the watch Add Expression box appears)

type *(int *)sine_a1

Repeat for variables:

*(int *)sine_a2

*(int *)phase_in

*(int *)speed_setpt

*(int *)dlog_iptr1

*(int *)dlog_iptr2

Allow continuous refresh of watch variable:

Right-mouse click on > Continuous refresh (in watch Window)

sine_a1, sine_a2 should now be incrementing (updating rapidly). The complete watch window is shown below –

# Adding Graph View

- Add Graph Window

Menu > View > Graph >Time/Frequency

Graph Property Dialog box will open.

Modify the following in Graph Property Dialog box –

- Start Address >		 0x8000

- Acquisition Buffer Size > 	400

- Display data Size > 		400

- DSP Data Type >		16-bit signed integer

The modified Graph Property Dialog box is shown below –

Click OK.

Right Click on Graph Window and

click on > Continuous refresh- as shown below -

Graph Window should be displaying  a Sine Wave at this time – as shown below.

- Add a second Graph Window similar to the first Window.

- The second Graph Window should have 0x8400 as its Starting Address as shown below –

- Click OK on Graph Property Dialog box for Second Graph Window.

- Right Click on Graph Window and  click on > Continuous refresh

- Second Graph Window should now be displaying a Cosine wave as shown below –

- The frequency of these waveforms can be changed by putting a different value for the variable speed_setpt in the Watch Window.

- Double-click on speed_setpt in watch Window to Edit the variable in the following Dialog Box –

- Change speed_setpt to 2000 and watch the graph windows change accordingly.

- The phase angle between these waves can be changed by modifying the varaible phase_in in the Watch Window.

- The following graph shows a waveforms at speed_setpt=2000 and phase_in=180 –

- The corresponding Watch Window is shown below –

- Real Time Tutorial for C24xx DSPs also provides work space example. In order to utilize example work space –

- Open work space EVM24xx.wks:

- Menu bar  > File > Workspace > Load Workspace

- Type

C:\tic2xx\c2000\tutorial\realtime\EVM24xx.wks

- The resulting workspace is shown below –