************************************

*				      *

*	 TMS340 Graphics Library      *

*      Notes for Release 2.01      *

*         October 30, 1990         *

*                                  *

************************************

This file contains release-specific updates to the published

documentation, including known software bugs and documentation

errors.

(1)  This release of the graphics library contains two ports to the

TMS34010 TIGA Development Board (TDB).  The first port, located

in directory \GLIB340\TDB10, supports a TDB with a full memory

configuration -- 1M of VRAM and 1M of DRAM.  The second port,

located in directory \GLIB340\TDB10MIN, supports a TDB with a

minimum memory configuration -- 512K of VRAM and no DRAM.

(2)  A batch file, CONFIG.BAT, is provided to automatically configure

the library to generate either TMS34010-compatible or TMS34020-

specific object code.  The MS-DOS commands are "CONFIG 34010"

and "CONFIG 34020", respectively.  As shipped, the object

libraries are configured for 34010-compatible code, which can

execute on both the 34010 and 34020, although it does not make

use of graphics acceleration features unique to the 34020.

(3)  An ASCII text version of the TMS340 Graphics Library User's

Guide is contained in this release in directory \GLIB340\MANUAL.

The contents of this directory are shipped as archive file

MANUAL.ZIP.  To de-archive the contents of this file, enter the

MS-DOS command "pkunzip -d manual.zip".

(4)  In the description of the get_fontinfo function on page 6-19 of

the TMS340 Graphics Library User's Guide, the charhigh field is

specified incorrectly as the sum of the ascent, descent, and

leading.  It should be specified as the sum of the ascent and

descent.

(5)  The descriptions of the clear_screen and clear_page functions in

the TMS340 Graphics Library User's Guide incompletely specify

the behavior of these functions.  Omitted from the manual is the

fact that each function clears the entire screen, ignoring the

current clipping window.  Also, the screen is cleared as though

the plane mask, pixel processing operation, and transparency

were set to their default values (0, 0, off), regardless of

their current settings.

(6)  The clear_page description in the user's guide incompletely

explains the potential problems with the use of the clear_page

function in double-buffered animation applications.  The

clear_page function may cause a portion of the screen to flicker

if it is called at any time other than the vertical blanking

interval.  This is because clear_page may -- depending on the

capabilities of the hardware -- use VRAM serial-register cycles

to rapidly clear a portion of the frame buffer and must

temporarily disable screen refreshing in order to do so.  These

problems can be avoided completely by calling the functionally

equivalent (but possibly slower) function, clear_screen, in

place of clear_page.  Another approach, which permits clear_page

to be used safely in a double-buffered application, relies on

the following inner loop structure:

draw_page = disp_page = 0;

for ( ; ; ) {

page_flip(disp_page, draw_page);

disp_page = draw_page;

draw_page = !draw_page;

< Perform background calculations. >

if (!page_busy())

clear_screen(-1);

else {

while (page_busy())

;

clear_page(-1);

}

< Draw to new page. >

}

In the code fragment above, if the background calculations delay

the clearing of the screen past the start of vertical blanking,

the clear_screen function is called in place of clear_page.

(7)  In the description of the encode_rect function on page 7-23 of

the TMS340 Graphics Library User's Guide, the last sentence in

the second paragraph from the bottom of the page is incorrect.

If the function is called with a bufsize of 0, the function

ignores the buf pointer and returns the number of bytes required

to store the compressed image.  In the following example, the

first call to encode_rect is used to obtain the size of the

image buffer required, and the second call actually encodes the

image:

unsigned long siz;

unsigned char *buf;

siz = encode_rect(w, h, x, y, 0, 0, 0);

if (!siz) {

buf = malloc(siz);

encode_rect(w, h, x, y, buf, siz, 0);

}

(8)  This release of the TMS340 Graphics Library was unable to track

some last-minute changes to the TIGA 2.0 MODEINFO data

structure.  The vram_block_write field in the structure that

follows the wksp_pitch field has been renamed the

silicon_capability field.  In addition, several new fields have

been appended to the structure to specify the color model,

including a color class parameter similar to that specified for

the X Window System.  Refer to Appendix A of the 1990 revision

of the TIGA User's Guide for details.  The TMS340 Graphics

Library may adopt the TIGA MODEINFO structure in a future

release.  Application programs that utilize the structure

definitions in the gsptypes.h include file should accommodate

such an update without requiring source code changes.

(9)  This release of the graphics library was unable to track some

last-minute changes to the TIGA 2.0 PAGE data structure.  The

single structure has been replaced in the new TIGA release by

two structures, PAGE10 and PAGE20, which are specific to the

TMS34010 and TMS34020 processors, respectively.  The TMS340

Graphics Library defines a single structure, PAGE, for both.

(10) The C shell utility, gspcl.exe, which is distributed with

releases 4.00 and 4.01 of the TMS340 Family Code Generation

Tools, has a bug that affects its use with the graphics library.

The -i (include file) option does not work correctly when

assembling *.asm source files.  Make sure that you use a later

version of the C shell utility that does not have this problem.

(11) Section 3.3 of the TMS340 Graphics Library User's Guide lists

the graphics library functions whose implementations (source

code files) differ from the corresponding TIGA implementations.

Two source files need to be added to this list:  the inittext.c

file in the \corprims directory and the text.c file in the

\extprims directory.  The inittext.c file contains the source

for the init_text function.  The text.c file contains the source

for the install_font, select_font, delete_font, set_textattr,

and get_textattr functions.  Only the init_text and install_font

functions differ from their TIGA counterparts.  The TMS340

Graphics Library versions of these functions use a statically

allocated font table that can accommodate up to 16 installed

fonts.  The TIGA versions of these functions use a dynamically

allocated font table that can grow as needed to accommodate more

fonts.  In most instances, the functional differences between

the TIGA and graphics library implementations of these functions

are of no consequence and can be ignored.

(12) The description of the set_transp function in the user's guide

states that the function is implemented only in TMS34020-based

systems.  The behavior in TMS34010-based systems is unspecified.

In fact, if set_transp is called in a TMS34010 system, the

function returns immediately, ignoring the mode argument, and

the transparency mode remains set at 0.  The graphics library

implementation of this function may differ slightly from the

version released with TIGA 2.0.

---- end of file ----